Doppler Radar Formulas

Dr. Brad Muller

Distance to a RADAR echo is given by the formula

R=cT2

where      R=range (distance to echo) 
                c = speed of electromagnetic radiation = 3 x 108 m s-1 
                T = time since pulse was emitted

Maximum unambiguous range:

Rmax=c2PRF

where    Rmax = maximum unambiguous range 
            c = 3 x 108 m s-1 
                PRF = pulse repetition frequency [s-1 ], i.e., number of pulses per second

The greater the frequency (more pulses per second), the shorter is the maximum unambiguous range.

Example:

If the NEXRAD is operating at a PRF of 326.09 s-1, what is the maximum unambiguous range?

Rmax=c2PRF=300,000,000 m s-12∙326.09 s-1=459996 m=460 km

Rmax=459996 m=460 km×0.539612 nm1 km=248.2 nm

_______________________

Fundamental sampling theorem:  To measure a frequency, fd, it is necessary to sample at a frequency of at least 2fd .

The sampling rate is the PRF, so

2fd=PRF

Doppler theorem:

V=-fλ2

where  f = frequency shift 
            = wavelength 
            V = radial velocity, i.e. component of velocity toward of away from the radar

Maximum unambiguous Doppler velocity is

Vmax=PRF∙λ4

Example:

If the NEXRAD is operating at a PRF of 326.09 s-1, what is the maximum unambiguous velocity?

First, convert NEXRAD wavelength from centimeters to meters:

λ=10 cm×1 m100 cm=0.10 m

Vmax=PRF∙λ4=326.09 s-1∙0.10 m4= 8.152 ms-1

Vmax= 8.152 ms-1×1.94384449 kt1 ms-1=15.8 kt

The "Doppler Dilemma:" Since the maximum unambiguous range is inversely related to the PRF while the maximum unambiguous Doppler velocity is directly related to the PRF, there is no single PRF that can maximize both at the same time.

Beam Height

The radar beam does not travel in a straight line in the earth’s atmosphere.  Under standard atmospheric conditions, it undergoes “standard refraction” or bending toward the earth’s surface:

Warning Decision Training Branch

The following material is directly from the Warning Decision Training Branch radar pages:

“WSR-88D Range Height Equation

The Range vs. Height diagram shown in Figure 11 is based on the WSR-88D Range Height Equation assuming standard refractive conditions. Equation (12) is the mathematical expression used to derive the height (h) curves in the diagram. For consistency, the height calculations generated by all WSR-88D algorithms use the following equation:

H = SR*sin PHI + (SR*SR)/(2*IR*RE)    Equation (12)

where

H = height of the beam centerline above radar level in km

SR = slant range in km

PHI = angle of elevation in degrees

IR = refractive index, 1.21

RE = radius of earth, 6371 km

For product generation, the metric units (km) are converted to nm and kft as necessary.”

Example:

For an elevation angle of 0.5 degrees, what is the beam height at a distance (slant range) of 90 nm?

First convert the distance to km:

SR= 90 nm×1.852 km1 nm=166.68 km

H=SR∙sinPHI+SR22∙IR∙RE=

=166.68 km∙sin0.5+166.68 km22∙1.21∙6371 km=3.256 km

H=3.256 km×3280.8 ft1 km=10684 ft